KEY FINDINGS:

- We decide the sensor, focal length and FOV. Resolution and WD are calculated from these three things.

- Sensor pixel count  (from camera datasheet)

- Focal length  (which lens we buy)

- FOV  (how large a physical area each patch covers)

- Working Distance  =  (FOV x Focal Length) / Sensor Size

- px/mm  =  Sensor Pixels (H) / FOV (H in mm)

- Focal length does not control resolution. Changing the focal length only changes the working distance. If FOV is constant and we change the focal length, px/mm stays identical.

- Example:

Hikrobot 20MP camera, 35mm lens, 30 patches covering a 150mm x 150mm board.

| Sensor pixel count (H) | 5496 pixels | Hikrobot MV-CS200-10GM datasheet — Sony IMX183 sensor |
| --- | --- | --- |
| Sensor pixel count (V) | 3672 pixels | Hikrobot MV-CS200-10GM datasheet |

| Focal length | 35mm | Lens (example) — Computar M3520-MPW2 |
| --- | --- | --- |
| FOV width | 35mm | Calculated from patch requirement — this is how much physical board each patch must cover |

Working Distance:  WD  =  (FOV x Focal Length) / Sensor Size (mm)

Substituting values:  WD  =  (35mm  x  35mm) / 13.3mm  =  92mm

The camera lens must be placed exactly 92mm above the PCB surface to capture a 35mm wide patch        on this sensor with this focal length.

Resolution (px/mm):  px/mm  =  Sensor Pixels (H) / FOV (H in mm)

Substituting values:  px/mm  =  5496 pixels / 35mm  =  157 px/mm

- If FOV is more, number of pixels/mm(resolution) decreases. Resolution completely depends on how much FOV we cover during capturing of image.

- FOV is not arbitrary. It is determined by how many patches we want and how large our board is.

- Example:

Given: Board = 150mm x 150mm,  patches = 30.

5 x 6 grid (5 columns x 6 rows = 30 patches).

| Direction | Board size | Number of steps | Step size | FOV per patch (with 20% overlap) |
| --- | --- | --- | --- | --- |
| Horizontal (X) | 150mm | 5 columns | 150 / 5 = 30mm | 30 / 0.8 = 37.5mm |
| Vertical (Y) | 150mm | 6 rows | 150 / 6 = 25mm | 25 / 0.8 = 31.25mm |

FOV= 35mm X 30mm

- FOV is derived from patch count requirement. Board size and max patches together fix the FOV. FOV then fixes px/mm and WD through the formulas.

CONCLUSION:

- Sensor Pixel Count

More megapixels means more px/mm at the same FOV. The FOV and WD stay unchanged — only the resolution changes.

| Camera | Sensor pixels (H) | FOV | px/mm achieved |
| --- | --- | --- | --- |
| Hikrobot 5MP (IMX264) | 2448 px | 35mm | 2448 / 35 = 70 px/mm |
| Hikrobot 12MP (IMX226) | 4024 px | 35mm | 4024 / 35 = 115 px/mm |
| Hikrobot 20MP (IMX183) | 5496 px | 35mm | 5496 / 35 = 157 px/mm |
| Lucid Atlas10 (IMX925) | 5328 px | 35mm | 5328 / 35 = 152 px/mm |

FOV and WD stay the same for all four cameras above — only the pixel count per mm changes. Buying a higher MP camera is the cleanest way to increase resolution without changing our physical setup.

- FOV

A smaller FOV means fewer mm per patch — so the same number of pixels cover a smaller physical area, giving more px/mm.

| FOV per patch | px/mm (20MP sensor) | Patches for 150mm board (20% overlap) |
| --- | --- | --- |
| 50mm | 5496 / 50 = 110 px/mm | ceil(150/40) x ceil(150/40) = 4x4 = 16 |
| 35mm | 5496 / 35 = 157 px/mm | 5x6 = 30 |
| 25mm | 5496 / 25 = 220 px/mm | ceil(150/20) x ceil(150/20) = 8x8 = 64 |
| 20mm | 5496 / 20 = 275 px/mm | ceil(150/16) x ceil(150/16) = 10x10 = 100 |

- Focal Length

Focal length does not change px/mm. If we keep FOV constant and change only the focal length, the working distance changes but resolution stays identical.

| Focal length | FOV (constant) | WD = (FOV x FL) / Sensor | px/mm |
| --- | --- | --- | --- |
| 25mm | 35mm | (35 x 25) / 13.3 = 66mm | 5496 / 35 = 157 px/mm |
| 35mm | 35mm | (35 x 35) / 13.3 = 92mm | 5496 / 35 = 157 px/mm |
| 50mm | 35mm | (35 x 50) / 13.3 = 131mm | 5496 / 35 = 157 px/mm |
| 75mm | 35mm | (35 x 75) / 13.3 = 197mm | 5496 / 35 = 157 px/mm |