PCB Defect Detection

Camera & Resolution

1  Physical Dimensions — PCB & Defects

## 1.1  PCB Board Dimensions

The test PCB used in this has the following physical dimensions:

- Board size: 150mm × 150mm  (15cm × 15cm)

## 1.2  Defect Physical Sizes

The table below lists all defect classes with their physical sizes, based on IPC-A-610 industry standards.

| Defect Class | Physical Size | Category | Visible from Top? |
| --- | --- | --- | --- |
| Solder ball | 0.1 – 0.5 mm | Solder | Yes |
| Dry solder / Cold joint | 0.3 – 1.0 mm | Solder | Yes (texture diff) |
| Solder bridge | 0.05 – 0.3 mm | Solder | Yes |
| Excess solder | 0.5 – 2.0 mm | Solder | Yes |
| Insufficient solder | 0.3 – 1.0 mm | Solder | Yes (texture diff) |
| Pad damage | 0.5 – 2.0 mm | Board | Yes |
| Open circuit (trace) | 0.1 – 0.5 mm | Board | Yes |
| Tombstone | 1.0 – 5.0 mm | Component | Partial — needs oblique |
| Component liftup | 0.5 – 3.0 mm | Component | Partial — needs oblique |
| Missing component | 0.5 – 10 mm | Component | Yes |
| Wrong component | 0.5 – 10 mm | Component | Yes |
| Misalignment | 0.1 – 2.0 mm | Component | Yes |
| Polarity reversal | 1.0 – 5.0 mm | Component | Yes |
| Foreign object | 0.5 – 5.0 mm | Contamination | Yes |
| Scratch / Trace damage | 0.2 – 3.0 mm | Board | Yes |

2  Resolution Requirements — All Calculations

## 2.1  The Core Rule

Resolution is always expressed as px/mm (pixels per millimetre) — how many pixels cover 1mm of physical space on the PCB.

Rule: we need at least 10 pixels across the smallest defect for a CNN / YOLO model to reliably detect and classify it.

Formula: Minimum px/mm  =  10 pixels ÷ smallest defect size (mm)

Key: For a 1mm smallest defect:  min px/mm = 10 ÷ 1 = 10 px/mm (absolute floor).  Recommended working target = 30–50 px/mm for a comfortable margin.

## 2.2  Why 10 px/mm Is Not Enough in Practice

At exactly 10 px/mm a 1mm defect is only a 10×10 pixel region. This causes several problems:

- Defects near a tile edge lose pixels — effective coverage drops to 5–7 px.

- Classification needs texture inside the bounding box. A 10px region is nearly a uniform blob — no learnable features.

- YOLOv8 bounding boxes include surrounding context. At 10px the entire detection region is tiny.

- Real defects are irregular shapes. Below 20px, shape information is completely lost.

The practical minimum for meaningful ML detection is 20–30 px/mm. 50 px/mm is the comfortable working target.

## 2.3  px/mm vs. Defect Size in Pixels

| px/mm | 1mm defect | 0.5mm defect | 0.3mm defect | Verdict |
| --- | --- | --- | --- | --- |
| 10 | 10 × 10 px | 5 × 5 px | 3 × 3 px | Absolute minimum — risky |
| 20 | 20 × 20 px | 10 × 10 px | 6 × 6 px | Marginal for 1mm defects |
| 30 | 30 × 30 px | 15 × 15 px | 9 × 9 px | Acceptable — recommended minimum |
| 50 | 50 × 50 px | 25 × 25 px | 15 × 15 px | Good — comfortable working target |
| 100 | 100 × 100 px | 50 × 50 px | 30 × 30 px | Excellent — safe for all defects |

## 2.4  Full Board Resolution

Board size: 150mm × 150mm. Required total pixel count for the full board at each px/mm target:

| Target px/mm | Pixels (H) | Pixels (V) | Total MP | Single Shot Feasibility |
| --- | --- | --- | --- | --- |
| 10 px/mm | 1,500 px | 1,500 px | ~2.25 MP | Any camera — trivial |
| 20 px/mm | 3,000 px | 3,000 px | ~9 MP | 12MP+ camera — single shot |
| 30 px/mm | 4,500 px | 4,500 px | ~20 MP | 20MP+ camera — single shot |
| 50 px/mm | 7,500 px | 7,500 px | ~56 MP | Stitching required |
| 100 px/mm | 15,000 px | 15,000 px | ~225 MP | Stitching required |

Key: At 30 px/mm, a 20MP camera (5496 × 3672 px) covers the full 150×150mm board in one shot: 5496 ÷ 30 = 183mm > 150mm. No stitching needed.

## 2.5  Tile Count for Stitching (12MP and 20MP Cameras)

| Camera | px/mm | Tile FOV | Tiles for 150×150mm | Stitching? |
| --- | --- | --- | --- | --- |
| 12MP (4024×3036) | 10 | 402 × 303 mm | 1 tile | No |
| 12MP (4024×3036) | 30 | 134 × 101 mm | 2×2 = 4 tiles | Yes |
| 12MP (4024×3036) | 50 | 80 × 60 mm | 2×3 = 6 tiles | Yes |
| 12MP (4024×3036) | 100 | 40 × 30 mm | 4×5 = 20 tiles | Yes |
| 20MP (5496×3672) | 10 | 549 × 367 mm | 1 tile | No |
| 20MP (5496×3672) | 30 | 183 × 122 mm | 1×2 = 2 tiles | Barely |
| 20MP (5496×3672) | 50 | 110 × 73 mm | 2×3 = 6 tiles | Yes |
| 20MP (5496×3672) | 100 | 55 × 37 mm | 3×5 = 15 tiles | Yes |

3  Full Board vs. Component-Level Patches

## 3.1  Full Board Capture

A single image (or stitched composite) covering the entire 150×150mm PCB.

Advantages

- Simple pipeline — one image per PCB, no crop location decisions needed.

- Captures spatial context and component positions across the board.

- Works well for large defects: missing component, wrong component, polarity reversal.

Disadvantages

- At 30 px/mm, a 1mm defect is 30×30 px within a 4500×4500 px image — occupies 0.04% of the area, making detection harder.

- ML models struggle with extreme scale imbalance between small defects and large image.

## 3.2  Component-Level Patch Capture

The board is divided into a grid of smaller crop regions, each covering one component or a small zone. Each patch is fed independently to the ML model.

Advantages

- Defects occupy a much larger fraction of each patch — easier for the model to detect.

- A 20×20mm patch at 50 px/mm = 1000×1000 px — feasible with any camera.

Disadvantages

- Defects at patch boundaries may be split across two crops.

4  Camera Positioning — Fixed vs. Moving, Views Needed

## 4.1  Fixed Camera

One camera mounted directly above the PCB. The PCB sits on a flat surface or motorised stage below.

- Simple setup — no moving parts on the camera side.

- No alignment errors — camera position is fixed and repeatable.

- At 30 px/mm with a 20MP camera, a single shot covers the full board, but as discussed above tiny defects may be hard to detect.

## 4.2  Fixed Camera + Motorised Stage (For Stitching)

Camera stays fixed. A motorised stage moves the PCB through a grid of positions, capturing one tile per position.

## 4.4  How Many Views Are Needed?

| Defect Type | Top-Down View | Oblique / Side View | Notes |
| --- | --- | --- | --- |
| Missing component | Sufficient | Not needed | Clearly visible from top |
| Wrong component | Sufficient | Not needed | Marking / colour visible |
| Solder ball | Sufficient | Not needed | Bright dot from top |
| Dry solder | Sufficient | Not needed | Texture difference visible |
| Solder bridge | Sufficient | Not needed | Bridge between pads visible |
| Pad damage | Sufficient | Not needed | Surface damage visible |
| Misalignment | Sufficient | Not needed | Offset from pad visible |
| Polarity reversal | Sufficient | Not needed | Marking orientation visible |
| Tombstone | Partial only | Required (30–45°) | Lifted end looks normal from top |
| Component liftup | Partial only | Required (30–45°) | Gap under component invisible from top |

5  Image Stitching — When and How

## 5.1  When Is Stitching Needed?

Stitching is required when a single camera frame cannot cover the full PCB at the desired resolution:

- 20MP camera at 30 px/mm → single shot covers full board  (NO stitching needed)

- 20MP camera at 50 px/mm → 6 tiles needed  (stitching required)

| Camera Model | Resolution | Single shot at 30 px/mm | Tiles at 50 px/mm | Tiles at 100 px/mm |
| --- | --- | --- | --- | --- |
| Hikrobot MV-CE120-10GM | 4024x3036 (12MP) | No | 6 tiles | 20 tiles |
| Hikrobot MV-CE200-10GM | 5472x3648 (20MP) | Yes | 6 tiles | 15 tiles |
| Basler ace2 Pro 12MP | 4024x3036 (12MP) | No | 6 tiles | 20 tiles |
| Basler ace2 Pro 20MP | 5496x3672 (20MP) | Yes | 6 tiles | 15 tiles |
| Teledyne FLIR Blackfly S 20MP | 5472x3648 (20MP) | Yes | 6 tiles | 15 tiles |
| Allied Vision Alvium 20MP | 5496x3672 (20MP) | Yes | 6 tiles | 15 tiles |
| XIMEA xiC 20MP | 5496x3672 (20MP) | Yes | 6 tiles | 15 tiles |
| Prosilica GT 4090 | 4096x3072 (12.5MP) | No | 5 tiles | 12 tiles |
| Lucid Vision Atlas10 | 5328x4608 (24.5MP) | Yes | 4 tiles | 9 tiles |

- 2MP camera at 30 px/mm → 4 tiles needed  (stitching required)

- Any camera at 100 px/mm → 15–20 tiles  (stitching required)

- Research Papers To Support the Analysis

- "Online PCB Defect Detector On A New PCB Defect Dataset" Tang, He, Huang, Yang — Shanghai Jiao Tong University, 2019 arXiv: 1902.06197

https://arxiv.org/pdf/1902.06197

The DeepPCB dataset, one of the most widely used benchmarks for PCB defect detection, was captured using a linear scan CCD at a resolution of approximately 48 px/mm. The full-board images are approximately 16,000 × 16,000 pixels, which are then cropped into 640×640 patches for model training. This directly validates the 30–50 px/mm target chosen for this project, as it demonstrates that this resolution range is the practical standard adopted in peer-reviewed PCB inspection research.

- "Attentive Context and Semantic Enhancement Mechanism for PCB Defect Detection" Scientific Reports, Nature, August 2024 nature.com/articles/s41598-024-69207-8

https://www.nature.com/articles/s41598-024-69207-8

This paper experimentally demonstrates that increasing input image resolution from 640×640 to 800×800 pixels leads to measurable improvements in both average precision and average recall for PCB defect detection. The authors attribute this gain to more context information being available to the model and better handling of defects appearing at different scales. This supports the choice of a higher px/mm target, as more pixels per defect directly improve detection accuracy.

- “Component Size-Resolution Matching”  ZhenHuaXing AOI Selection Guide aoi-spi.com

https://aoi-spi.com/choose-automatic-optical-inspection/

The resolution of AOI for 01005 components should be higher than 12 µm/pixel, while for 0402 components it should be higher than 25 µm/pixel. Resolution is calculated as FOV divided by the camera pixel count. A 0402 component (1.0mm × 0.5mm) requires 25 µm/pixel = 40 px/mm — directly supporting the 30–50 px/mm recommendation for a 1mm defect threshold.

- "An Improved Algorithm for Small Object Detection Based on YOLOv4" Computers and Electrical Engineering, 2023 sciencedirect.com/science/article/pii/S0045790622007054

https://www.sciencedirect.com/science/article/abs/pii/S0045790622007054

This paper identifies limited image resolution as one of the primary reasons YOLO-family models miss or incorrectly detect small objects, noting that unclear features at low resolution degrade classification accuracy. The authors propose architectural modifications specifically to compensate for this resolution deficiency. This directly supports the argument that resolution must be set correctly at the capture stage rather than relying on model-side compensations.

- TRI White Paper: Introduction to AOI Technology Test Research Inc. (TRI)

https://www.circuitnet.com/news/uploads/2/WP_TRI_Introduction_to_AOI_CN.pdf

10 µm system resolution means each camera pixel corresponds to a 10 µm square of the PCB, and as a result 1mm of the PCB is represented by 100 pixels. Choosing a suitable system resolution is one of the key parameters when selecting an AOI solution, typically involving required inspection speed, inspection accuracy, and size of the smallest components. 10 µm/pixel = 100 px/mm is the standard used in high-end industrial AOI machines for fine-pitch components. For 1mm defects, 30–50 px/mm (33–20 µm/pixel) is perfectly aligned with where the industry operates for standard components.